uCal_Driver_Fix

Step 1: Go to ‘Start” (Window Symbol in the lower right corner of screen), then ‘Document’, then click on ‘This PC’ (Half-way down on the left).

Step 2: Click on ‘Manage’ (Middle Top of Screen), see above.

Step 3: Click on ‘Device Manager’, see below.

Step 4: Right Click on ‘uCal Device….’ Under “Other Devices’. See above.

NOTE: ucal…. Should be under ‘Universal Serial Bus controllers’.

Step 5: Click on ‘Update Driver’, see below.

Step 6: Click ‘Browse my computer for driver software’, see below.

Step 7: Click ‘Let me pick from a list of available drivers on my computer’, see below.

Note: Select the ‘uCal_G25 WHQL USB Drivers’ Folder when using uCal_G25 (uCal w/ LED Light).

Step 8: Click on ‘Show All Devices’, then click on ‘Next’, see below.

Step 9: Click on ‘Have Disk’, see below.

Step 10: Click On ‘Browse’, See below.

Step 11: Navigate to: ‘C:\Program Files (x86)\uCal\uCal WHQL USB Drivers’, see above.

Step 12: Click on ‘ftdibus’, then click on ‘Open’, see below.

Step 13: Click on ‘OK’, see below.

Step 14: Click on ‘USB Serial Converter’, then click on ‘Next’, see below.

Step 14: Click on ‘Yes’, see below.

Step 15: Click on ‘Close’, see below.

Step 16: Click on ‘Close’, see below.

Step 17: uCal, i.e. ‘USB Serial Converter’ now shows up under ‘Unversal Serial Bus controllers’, which is the correct location, see below. Close down all open windows, and proceed as usual. Done.